Water (or liquid) Level and its Temperature Monitoring System with graphics and animation using Arduino and TFT LCD

Introduction:

Water or liquid level and its temperature monitoring system is the most essential system that is found in all most all chemical, pharmaceutical, textile or many such industries. Liquid level and temperature monitoring system plays vital role in maintaining required liquid level and also maintaining its temperature constant.

For example, in chemical industry, maintaining liquid level and its temperature at required set value is crucial for the quality of final product. So, they are constantly monitored during entire process.

This system comprises of contact type or noncontact type water level senor and probe type temperature sensor with digital display that continuously monitors level and temperature of liquid or water and give real time data.

All currently available water or liquid level and temperature monitoring systems uses simple LED indications or 7-segment type display. All such displays just indicate water level or temperature in form of LED bar-graph. They don’t display actual amount of water (in liter) or actual temperature value in degree centigrade. Few modern systems comprise of simple text LCD type display that can display actual amount of water level in liter or temperature in degree centigrade.

This project demonstrates water level and its temperature monitoring on 4.3” TFT LCD with graphics and color changing animation of water filling up in tank and temperature rising in bar graph. The following figure shows the snap shot of TFT display.

As shown in above figure, the water level in tank is displayed in liter as well as in % that is actual level of water in tank. This water level is also displayed with real time color changing animation of water filling up in tank as shown. Similarly, the actual water temperature is displayed in degree centigrade with real time color changing bar-graph animation.

This is completely new and innovative way of monitoring water level and its temperature using multicolor graphics and animation.

The project utilizes HCSR04 UDM (ultrasonic distance measurement) sensor module, water temperature sensor probe DS18B20 along with Arduino NANO board. It features a 4.3” SPI TFT LCD with a resolution of 480x320 pixels to display actual water level in liter and water temperature.

Block diagram of system:

The figure shows the block diagram of the system and its major building blocks.

The major building blocks are ultrasonic distance measurement (UDM) sensor HC-SR04, water temperature sensor DS18B20, SPI TFT display and Arduino NANO board.

UDM sensor HC-SR04:

This sensor is used to measure water level inside tank. It transmits ultrasonic sound waves of 40KHz and receives echo of reflected waves from water level. It gives PWM output as pulse width that is inversely proportional to the water level inside tank.

Water temperature sensor probe DS18B20:

DS18B20 is water temperature sensor probe from Dallas. It works on one-wire protocol and it gives direct digital value of water temperature in degree centigrade.

SPI TFT LCD:

Its 4.3” TFT LCD with resolution of 480x320. It works on SPI. It is used to display water level in liter as well as in % and water temperature in degree centigrade. It displays both water level and temperature in multicolor with full graphics and animation.

Arduino NANO board:

It’s a main building block as MCU (microcontroller unit) of the system. It performs following tasks

- Measures pulse width output from HCSR04 sensor and calculates water level.

- Reads digital output form DS18B20 sensor of water temperature.

- Display both water level and temperature on TFT LCD screen with full graphics and animation.

Circuit diagram:

As shown in figure, the circuit is built using Arduino NANO board, HCSR04 sensor, DS18B20 probe, TFT LCD and some resistors.

- Sensor probe DS18B20 has 3 interfacing pins. (1) +V (2) op and (3) Gnd. +V pin is connected to 5 V output from Arduino board. Gnd pin is connected to circuit ground. Op pin is connected to digital pin 5 (D5). One 4.7K pull up is connected between +V and op pin as it is recommended in the datasheet for DS18B20 sensor.

- The TFT LCD has 9 interfacing pins. These pins are connected with Arduino board pins as given in the table.

| TFT LCD Pin | Arduino NANO pin |
| --- | --- |
| Vcc | 5 V |
| Gnd | Gnd |
| LED | 3.3 V |
| CS | D8 |
| RST | D9 |
| DS | D10 |
| SDO | D11 |
| SDI | D12 |
| SCK | D13 |

- There is voltage divider network (build using 1K and 2.2K resistors) between Arduino and TFT LCD pins as shown in the circuit. This is required as TFT LCD pins works on 3.3V and Arduino digital pins gives 5V output. So, this voltage divider network arrangement converts 5 V output of Arduino pins into 3.3V.

- HCSR04 sensor has 4 interfacing pins. (1) Vcc (2) Trig (3) Echo and (4) Gnd. Vcc pin is connected to 5V output pin of Arduino board and Gnd pin is connected with circuit ground. Trig and echo pins are connected to digital pins D4 and D3 of Arduino board respectively.

Here is the snap of circuit connections and arrangement of components

The following figure shows suggested arrangements of circuit components and its housing.

Working and operation:

Measuring water temperature:

The water temperature is measured using sensor probe DS18B20. As it is already told, the DS18B20 works on one wire protocol and it gives direct value of water temperature in degree centigrade. This value is read by Arduino nano and it is directly displayed on TFT LCD screen.

Measuring water level:

As everyone knows, the property of ultrasonic sound waves that it gets reflected by water surface. The HCSR04 sensor works on the principle of transmission and reflection of ultrasonic waves. So, using this property and the working principle of HCSR04, we can measure the water level inside tank.

- The Arduino apply trigger pulse (trig ip) of 10 uS to HCSR04.

- On getting trigger input, the HCSR04 transmits burst pulses of 40 KHz (ultrasonic sound).

- These pulses strike with water surface (level) inside tank and gets reflected.

- HCSR04 sensor receives these reflected pulses and calculates the to-n-fro traveling time of pulses.

- Based on traveling time it will generate PWM output on its echo pin as pulse width directly proportional to traveling time.

- This width of pulse is measured by Arduino nano board and it calculates the distance of water surface (level) from sensor.

- For example, if water surface distance is measured as 10 cm and total depth of water tank is 30 cm, then water level is measured as 30 – 10 = 20 cm

- Further, this 20 cm is calibrated in terms of liter. It is as follows.

Total capacity of water tank is 15 ltr and its depth is 30 cm.

So, we can say

2 cm water level = 1 liter water

From this we can easily calculate water level in liter from water level in centimeter.

Displaying water level and temperature on TFT LCD with graphics and animation:

The Arduino uses “LCDWIKI_GUI” and “LCDWIKI_SPI” libraries to display water level and temperature on 480x320 pixel SPI TFT LCD screen. Using these two libraries Arduino displays text, create graphic for water tank and temperature bar graph and also generates color changing animation like filling water in tank or filling temperature bar graph.

The water level in tank is displayed in liters from 0 to 15 with calibration and also in % from 0 to 100. It is also displayed graphically as water tank and water filling animation as 25%, 50%, 75% and so on. The fill water color changes to RED if water level is <10% and it changes to full BLUE from CYAN when water level is > 80%.

The temperature is displayed in degree centigrade from 20 to 70 oC. The animated bar-graph displays the temperature as 35 oC, 45 oC, 60 oC ,70 oC etc. The bar-graph color changes from BLUE – CYAN – YELLOW to RED as temperature increases.

Software program and logic:

The circuit operation is due to the software program downloaded into internal FLASH memory of Arduino board microcontroller ATMega328. The program is written in Arduino C/C++ language. It is compiled and build using Arduino IDE software and uploaded into FLASH memory of ATMega328.

The program uses following four libraries

- DallasTemperature.h – this library is used to read data from DS18B20 sensor

- OneWire.h – this library is used to initialize one-wire protocol and communicate with DS18B20 sensor

- LCDWIKI_GUI.h – this library is used to display characters, numbers, shapes, animation etc and create GUI on TFT display

- LCDWIKI_SPI.h - this library is used to communicate with SPI type TFT display using 4 wire SPI protocol